AUTOMOTIVE FUNCTIONAL SAFETY FMEA RISK LENS Evidence-based scoring for automotive electronics design reviews S IMPACT; O RECURRENCE; D CONTROL CONFIDENCE. A custom engineering framework by Hamza Alzakarneh Automotive Functional Safety Engineering Playbook

ORIGINAL PROJECT AID  •  VERSION 1.0

This document is not an official ISO, AIAG-VDA, OEM, or regulatory ranking table. Tailor it to the project and approved organizational process.

| 1 | FRAMEWORK IN ONE VIEW The score starts the discussion; the evidence and response lane drive the decision. |
| --- | --- |

The FMEA Risk Lens keeps the familiar Severity–Occurrence–Detection model but requires a written evidence basis beside every rating. It is designed for automotive electronics teams that need a direct connection between design risk, safety architecture, diagnostics, validation, and closure evidence.

| SEVERITY What is the highest credible effect? | OCCURRENCE How credible is the cause in this design? | DETECTION How confidently do controls prevent or detect it? |
| --- | --- | --- |

Four response lanes

| A — STOP & ESCALATE | Severity 9–10, safety-goal concern, or uncontrolled hazardous response. |
| --- | --- |
| B — ARCHITECTURE ACTION | High impact with weak control, single-point dependency, independence gap, or FTTI concern. |
| C — DESIGN ACTION | Material design/process risk, incomplete verification, adverse trend, or project action trigger. |
| D — CONTROL & MONITOR | Low impact with stable prevention and detection evidence and no escalation trigger. |

RPN = Severity × Occurrence × DetectionUse RPN only to sort comparable items. High severity, safety-goal linkage, weak independence, or timing violations override the arithmetic score.

| 2 | SEVERITY — IMPACT LENS Rate the highest credible effect. Stronger inspection does not reduce severity. |
| --- | --- |

| # | LEVEL | RATING CRITERION | EVIDENCE PROMPT |
| --- | --- | --- | --- |
| 10 | Critical | Potential life-threatening injury or uncontrolled hazardous vehicle response with little practical opportunity for mitigation. | Hazard linkage; vehicle effect; safety-goal relevance. |
| 9 | Severe | Potential serious injury, loss of a safety-critical function, or major safety-related noncompliance. | Safety analysis; compliance interface; warning assumptions. |
| 8 | Primary loss | Primary function is lost, the product becomes inoperable, or immediate controlled shutdown is required. | Functional architecture; safe/degraded state. |
| 7 | Major degradation | Major function is significantly degraded, continued use is restricted, or immediate service is required. | Vehicle/customer effect; operating limitation. |
| 6 | Functional reduction | Subsystem remains available with a clear performance, availability, or operating-range reduction. | Measured boundary; requirement deviation. |
| 5 | Customer disruption | Function remains available, but customer experiences a clear loss of performance, convenience, or confidence. | Customer-visible symptom; requirement comparison. |
| 4 | Minor deviation | Limited performance deviation exists and a practical workaround is available without material risk. | Workaround; performance evidence. |
| 3 | Nuisance | Effect is noticeable but does not meaningfully reduce the intended function. | Observable symptom; no-loss rationale. |
| 2 | Trace effect | Effect is barely perceptible and does not change normal use. | Measurement or tolerance comparison. |
| 1 | No meaningful effect | No credible customer, vehicle, manufacturing, or downstream functional effect is identified. | Containment or irrelevance analysis. |

Key rule: Severity follows the effect at the next level and vehicle/customer level—not the local component symptom and not the strength of the current control.

| 3 | OCCURRENCE — RECURRENCE LENS Rate the credibility of the cause using design, test, process, supplier, reliability, and field evidence. |
| --- | --- |

| # | LEVEL | RATING CRITERION | EVIDENCE PROMPT |
| --- | --- | --- | --- |
| 10 | Persistent | Cause is active in normal conditions and the failure is routinely reproduced. | Repeated normal-condition test or field evidence. |
| 9 | Very frequent | Cause is commonly encountered and requires little combination of conditions. | High-frequency data; weak margin; dominant mechanism. |
| 8 | Recurrent | Failure repeats across builds, units, environments, or operating cycles. | Multi-unit reproduction; confirmed cause. |
| 7 | Frequent | Failure occurs in a defined but regularly encountered operating scenario. | Scenario exposure; reproduced mechanism. |
| 6 | Occasional | A credible cause has appeared in test, production, supplier, or field evidence. | Documented events; relevant configuration. |
| 5 | Intermittent | Cause is credible but requires a limited combination of conditions or has sparse evidence. | Boundary test; early-life data; mechanism analysis. |
| 4 | Infrequent | Cause requires multiple contributing conditions or is strongly reduced by design/process margin. | Margin analysis; process capability. |
| 3 | Rare | Cause is physically credible only near unusual or extreme conditions. | Stress analysis; rare-condition exposure. |
| 2 | Highly unlikely | Mechanism is difficult to realize within the validated operating envelope. | Robust margin; no observed events. |
| 1 | Remote | No credible cause remains after design review, analysis, and representative validation. | Documented prevention evidence. |

Key rule: Do not transfer an occurrence score from a generic component reputation. Score the cause in the actual architecture, stress envelope, process, and configuration.

| 4 | DETECTION — CONTROL-CONFIDENCE LENS Credit only controls with a defined fault population, operating range, timing, reaction, and verification evidence. |
| --- | --- |

| # | LEVEL | RATING CRITERION | EVIDENCE PROMPT |
| --- | --- | --- | --- |
| 10 | No control | No prevention or detection control exists before exposure. | Explicit control gap. |
| 9 | Field-only | Issue is most likely discovered through complaint, field failure, or teardown. | Field-detection path only. |
| 8 | Weak sampling | Sampling or limited inspection may detect the issue, but escape probability remains high. | Sampling plan; blind spots. |
| 7 | Manual dependency | Detection depends heavily on operator judgment, visual inspection, setup, or exact conditions. | Work instruction; human-factor limitation. |
| 6 | Partial automated | Automated control exists but does not cover the complete fault population or operating range. | Coverage gaps; test limits; missed faults. |
| 5 | Moderate control | Controls detect many cases, but boundary, timing, or latent-fault gaps remain material. | Test evidence; residual escape conditions. |
| 4 | Robust detection | Automated detection is repeatable, limits are controlled, and reaction is defined. | Capability; calibration; thresholds; reaction. |
| 3 | Independent detection | Functionally independent monitor detects the fault within required time and controls the reaction. | Independence; timing capture; fault injection. |
| 2 | Multi-layer control | Independent prevention and detection layers cover the fault with periodic proof or self-test. | Layered coverage; proof-test interval; dependency review. |
| 1 | Prevention / near-certain | Cause is prevented by design or detected automatically with validated near-complete coverage and controlled reaction. | Prevention; validated coverage; configuration control. |

Key rule: “Diagnostics exist” is not evidence. Record what faults are covered, what remains, when detection occurs, and how the system reacts.

| 5 | WORKED AUTOMOTIVE ELECTRONICS EXAMPLE Inverter gate-driver shutdown path — from initial escalation to verified residual control. |
| --- | --- |

| FUNCTION | Inverter gate-driver shutdown path |
| --- | --- |
| FAILURE MODE | Shutdown command cannot disable one power stage |
| CAUSE | Shared enable path remains active following a component short |
| VEHICLE EFFECT | Torque may persist beyond the commanded removal interval |

| LENS | INITIAL | RATIONALE | RESIDUAL |
| --- | --- | --- | --- |
| Severity | 10 | Credible uncontrolled torque path can violate a propulsion safety goal. | 10 |
| Occurrence | 3 | Rare but physically credible short not eliminated by original architecture. | 2 |
| Detection | 8 | Primary controller lacks independent confirmation that the stage is disabled. | 2 |

Initial RPN: 10 × 3 × 8 = 240  •  Response Lane A — Stop & Escalate

Required engineering actions

Add an independent hardware disable path that does not share the primary enable node.

Monitor output-stage state using feedback independent of the commanded value.

Define and verify the complete shutdown timing budget.

Perform dependent-failure analysis on shared supply, ground, connector, clock, and PCB routing.

Inject open, short, stuck-active, and delayed-reaction faults; capture detection and torque-removal timing.

Review prompts before approval

Is the highest-level effect stated, or only the local symptom?

Does occurrence reflect this design and configuration?

Is the monitor independent enough to detect the fault it monitors?

Is detection and reaction inside the required fault-tolerant interval?

Is the residual score based on implemented and verified action?

Ownership statement: The FMEA Risk Lens, response lanes, scoring language, diagrams, and example are original material developed for the Automotive Functional Safety Engineering Playbook.